Kubernetes commands

installing kubernetes cluster

# Kubeadm Installation Guide

This guide outlines the steps needed to set up a Kubernetes cluster using kubeadm.

## Pre-requisites

- Ubuntu OS

- t2.medium instance type or higher

---

## AWS Setup

- Make sure all traffic is allowed on all instance

---

## Execute on Both "Master" & "Worker Node"

Run the following commands on both the master and worker nodes to prepare them for kubeadm.

=============================================================================================================

##Update the system's package list and install necessary things:

sudo apt-get update

sudo apt install apt-transport-https curl -y

## Install containerd

curl -fsSL https://download.docker.com/linux/ubuntu/gpg | sudo gpg --dearmor -o /etc/apt/keyrings/docker.gpg

echo "deb [arch=$(dpkg --print-architecture) signed-by=/etc/apt/keyrings/docker.gpg] https://download.docker.com/linux/ubuntu $(lsb_release -cs) stable" | sudo tee /etc/apt/sources.list.d/docker.list > /dev/null

sudo apt-get update

sudo apt-get install containerd.io -y

## Create containerd configuration

sudo mkdir -p /etc/containerd

sudo containerd config default | sudo tee /etc/containerd/config.toml

sudo sed -i -e 's/SystemdCgroup = false/SystemdCgroup = true/g' /etc/containerd/config.toml

sudo systemctl restart containerd

## Install Kubernetes

curl -fsSL https://pkgs.k8s.io/core:/stable:/v1.30/deb/Release.key | sudo gpg --dearmor -o /etc/apt/keyrings/kubernetes-apt-keyring.gpg

echo 'deb [signed-by=/etc/apt/keyrings/kubernetes-apt-keyring.gpg] https://pkgs.k8s.io/core:/stable:/v1.30/deb/ /' | sudo tee /etc/apt/sources.list.d/kubernetes.list

sudo apt-get update

sudo apt-get install -y kubelet kubeadm kubectl

sudo apt-mark hold kubelet kubeadm kubectl

sudo systemctl enable --now kubelet

## Disable swap

sudo swapoff -a

sudo modprobe br_netfilter

sudo sysctl -w net.ipv4.ip_forward=1

---------------------------------------------------

## Execute ONLY on "Master Node"

sudo kubeadm init --pod-network-cidr=10.244.0.0/16

mkdir -p $HOME/.kube

sudo cp -i /etc/kubernetes/admin.conf $HOME/.kube/config

sudo chown $(id -u):$(id -g) $HOME/.kube/config

#Network plugin

kubectl apply -f https://github.com/flannel-io/flannel/releases/latest/download/kube-flannel.yml

--------------------------------------------------------

## Execute on ALL of your Worker Node's

1. Perform pre-flight checks

sudo kubeadm reset pre-flight checks

2. Paste the join command you got from the master node and append `--v=5` at the end but first use sudo su command to become root (avoid using sudo your-token).

sudo su

<your-token --v=5>

## Verify Cluster Connection

**On Master Node:**

kubectl get nodes

----------------------------------------------------------------------

For K8 secret Manager:

kubectl create secret docker-registry ecr-secret   --docker-server=<Account-id>.dkr.ecr.us-east-2.amazonaws.com   --docker-username=AWS   --docker-password=$(aws ecr get-login-password --region <region>)   --namespace default

—----------------------------------------------------

for project - 2 to install docker this need to be done :

The error indicates that containerd.io conflicts with another package (containerd) on your system, or that there are held packages causing dependency resolution issues. Here's how to resolve it step by step:

1. Check for Held Packages

Held packages can prevent updates or installations. Check for held packages using:

bash

Copy code

dpkg --get-selections | grep hold

If any packages are held, unhold them:

bash

Copy code

sudo apt-mark unhold <package-name>

2. Remove Conflicting Packages

Since containerd.io conflicts with containerd, you may need to remove containerd:

bash

Copy code

sudo apt-get remove --purge containerd

3. Clean Up and Update

Clean up and refresh the package manager:

bash

Copy code

sudo apt-get autoremove

sudo apt-get autoclean

sudo apt-get update

4. Install containerd.io

Try installing containerd.io again:

bash

Copy code

sudo apt-get install containerd.io

5. Force Installation (if Necessary)

If conflicts persist, force the installation (use with caution):

bash

Copy code

sudo apt-get install -f containerd.io

6. Use Alternative Installation Method

If the issue persists, you can download and manually install the .deb package:

Download the containerd.io package from the official Docker repository.

Install it manually:

bash

Copy code

sudo dpkg -i containerd.io_<version>.deb

sudo apt-get -f install

7. Check Docker and Containerd Versions

If you're using Docker, ensure the versions of Docker and containerd.io are compatible. Update Docker if necessary:

bash

Copy code

sudo apt-get install docker-ce docker-ce-cli

If these steps don't resolve the issue, let me know! Providing the output of sudo apt-get install containerd.io can help diagnose further.